SIMATS ENGINEERING

ASSESSMENT TOOL 3

NAME:SIVARANGJENII B

REG NO: 192521375

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software:Cisco Packet Tracer,Wireshark

Annexure A

Debugging &amp; Optimisation Task

1. A network administrator has configured a workstation with the following settings:

 IP Address: 192.168.10.65

 Subnet Mask: 255.255.255.192 (/26)

 Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet

192.168.10.0/26.

Task

Debug the network configuration by:

 Identifying the subnet to which the configured IP address belongs.

 Determining the network address, broadcast address, and valid host range.

 Verifying whether the configured IP address and gateway are valid.

 Identifying the configuration errors.

 Optimizing the configuration by assigning an appropriate IP address and default gateway.

 Calculating the total number of usable host addresses after correction.

2. Two devices are configured with the following IPv6 addresses:

 Device A: 2001:db8::ff00:42:8329/64

 Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

 Expanding both IPv6 addresses into their complete notation.

 Determining the network prefix of each device.

 Verifying whether both devices belong to the same subnet.

 Identifying the addressing or prefix mismatch.

 Optimizing the IPv6 configuration by assigning appropriate addresses.

 Calculating the number of available host addresses within the corrected subnet.

3. An organization has allocated the network 192.168.1.0/24 into several departments using the

subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others

have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

 Identifying the network address, broadcast address, and host range for each subnet.

 Calculating the number of usable and unused IP addresses.

 Identifying inefficient subnet allocations.

 Debugging the addressing plan by locating subnet allocation mistakes.

 Recommending an optimized subnetting scheme using VLSM to improve address

utilization while supporting current departmental requirements.

4. Two devices are configured with the following IPv6 addresses:

 Device A: 2001:db8::ff00:42:8329/64

 Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

 Expanding both IPv6 addresses into their complete notation.

 Determining the network prefix of each device.

 Verifying whether both devices belong to the same subnet.

 Identifying the addressing or prefix mismatch.

 Optimizing the IPv6 configuration by assigning appropriate addresses.

 Calculating the number of available host addresses within the corrected subnet.

5. A router contains the following routing table.

Destination Network Next Hop

192.168.1.0/24 10.0.0.2

192.168.2.0/24 10.0.0.3

Default Route 10.0.0.1

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Task

Debug the routing configuration by:

 Identifying the matching routing entry using the longest-prefix match.

 Determining the next-hop router selected for packet forwarding.

 Verifying whether the destination IP belongs to the intended subnet.

 Identifying any routing configuration errors.

 Optimizing the routing table to improve packet forwarding efficiency.

 Identifying the route that will be used if no matching destination exists.

Question 1 – Debugging & Optimization of IPv4 Network Configuration

Given

- IP Address = 192.168.10.65

- Subnet Mask = 255.255.255.192 (/26)

- Default Gateway = 192.168.10.1

Step 1: Identify the Subnet

Subnet Mask = 255.255.255.192 (/26)

Block Size

= 256 − 192

= 64

Subnet Ranges:

- 192.168.10.0 – 192.168.10.63

- 192.168.10.64 – 192.168.10.127

- 192.168.10.128 – 192.168.10.191

- 192.168.10.192 – 192.168.10.255

The given IP address 192.168.10.65 falls within the range 192.168.10.64 – 192.168.10.127.

Therefore, the subnet is:

192.168.10.64/26

Step 2: Determine the Network Address

Network Address = 192.168.10.64

Step 3: Determine the Broadcast Address

The next subnet begins at 192.168.10.128.

Therefore,

Broadcast Address

= 192.168.10.127

Step 4: Determine the Valid Host Range

First Host

= 192.168.10.65

Last Host

= 192.168.10.126

Valid Host Range

= 192.168.10.65 – 192.168.10.126

Step 5: Verify the Configured IP Address

Configured IP Address

= 192.168.10.65

Since it lies within the valid host range, the IP address is valid.

Step 6: Verify the Default Gateway

Configured Gateway

= 192.168.10.1

This gateway belongs to subnet 192.168.10.0/26.

The workstation belongs to subnet 192.168.10.64/26.

Since they are in different subnets, the default gateway is invalid.

Step 7: Configuration Error

The configured default gateway belongs to a different subnet from the workstation.

Hence, the workstation cannot communicate with other devices in its subnet.

Step 8: Optimized Configuration

IP Address = 192.168.10.65

Default Gateway

= 192.168.10.66 (or any valid router IP between 192.168.10.65 and 192.168.10.126)

Step 9: Calculate the Total Number of Usable Hosts

Formula

= 2^(32 − 26) − 2

= 2^6 − 2

= 64 − 2

= 62 usable host addresses

Question 2 – Debugging & Optimization of IPv6 Configuration

Given

Device A

2001:db8::ff00:42:8329/64

Device B

2001:db8:0:0:1::1/64

Step 1: Expand the IPv6 Addresses

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Determine the Network Prefix

Prefix Length

= /64

Network Prefix for Device A

= 2001:db8:0:0::/64

Network Prefix for Device B

= 2001:db8:0:0::/64

Step 3: Verify Whether Both Devices Belong to the Same Subnet

Both devices have the same /64 network prefix.

Therefore, they belong to the same subnet.

Step 4: Identify the Addressing or Prefix Mismatch

There is no IPv6 addressing or prefix mismatch.

The communication failure may be caused by:

- Incorrect default gateway

- Firewall blocking traffic

- Interface disabled

- Duplicate IPv6 address

- Neighbor Discovery failure

Step 5: Optimize the IPv6 Configuration

Device A

= 2001:db8:0:0::10/64

Device B

= 2001:db8:0:0::20/64

Default Gateway

= 2001:db8:0:0::1

Step 6: Calculate Available Host Addresses

Host Bits

= 128 − 64

= 64

Available Addresses

= 2^64

= 18,446,744,073,709,551,616 addresses

Question 3 – Analyze and Optimize the Subnet Allocation

Given Network

192.168.1.0/24

Subnet Masks Used

- /26

- /27

- /28

Step 1: Analyze the /26 Subnet

Network Address

= 192.168.1.0

Broadcast Address

= 192.168.1.63

Valid Host Range

= 192.168.1.1 – 192.168.1.62

Usable Host Addresses

= 62

Step 2: Analyze the /27 Subnet

Network Address

= 192.168.1.64

Broadcast Address

= 192.168.1.95

Valid Host Range

= 192.168.1.65 – 192.168.1.94

Usable Host Addresses

= 30

Step 3: Analyze the /28 Subnet

Network Address

= 192.168.1.96

Broadcast Address

= 192.168.1.111

Valid Host Range

= 192.168.1.97 – 192.168.1.110

Usable Host Addresses

= 14

Step 4: Calculate Unused Addresses

If a department requiring only 10 hosts receives a /26 subnet, then

Unused Addresses

= 62 − 10

= 52 unused IP addresses

This indicates inefficient subnet allocation.

Step 5: Debug the Addressing Plan

Possible subnet allocation mistakes include:

- Assigning a large subnet to a department with very few users.

- Assigning a small subnet to a department requiring many hosts.

- Overlapping subnet ranges.

- Incorrect subnet boundaries.

Step 6: Recommend an Optimized VLSM Scheme

Assign subnet sizes according to department requirements.

Example:

HR Department (50 hosts)

→ /26

Sales Department (25 hosts)

→ /27

Finance Department (10 hosts)

→ /28

This VLSM allocation minimizes address wastage and improves network efficiency.

Question 4 – Debugging IPv6 Configuration

Given

Device A

2001:db8::ff00:42:8329/64

Device B

2001:db8:0:0:1::1/64

Step 1: Expand the IPv6 Addresses

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Determine the Network Prefix

Network Prefix

= 2001:db8:0:0::/64

Step 3: Verify Whether Both Devices Belong to the Same Subnet

Both devices belong to the same /64 network.

Step 4: Identify the Addressing Error

No IPv6 addressing mismatch exists.

Possible reasons for communication failure are:

- Incorrect default gateway

- Firewall restrictions

- Interface disabled

- Neighbor Discovery failure

Step 5: Optimize the Configuration

Device A

= 2001:db8:0:0::10/64

Device B

= 2001:db8:0:0::20/64

Gateway

= 2001:db8:0:0::1

Step 6: Calculate Available Host Addresses

Available Addresses

= 2^64

= 18,446,744,073,709,551,616 addresses

Question 5 – Debugging the Routing Configuration

Given Routing Table

192.168.1.0/24 → 10.0.0.2

192.168.2.0/24 → 10.0.0.3

Default Route → 10.0.0.1

Destination IP

= 192.168.2.100

Step 1: Identify the Matching Routing Entry

Destination IP

= 192.168.2.100

It matches the network

192.168.2.0/24

using the longest-prefix match.

Step 2: Determine the Next-Hop Router

Next Hop

= 10.0.0.3

Step 3: Verify the Destination Subnet

Network Address

= 192.168.2.0

Broadcast Address

= 192.168.2.255

Valid Host Range

= 192.168.2.1 – 192.168.2.254

The destination IP 192.168.2.100 lies within this range.

Therefore, the destination belongs to the intended subnet.

Step 4: Identify Routing Configuration Errors

Possible routing errors include:

- Incorrect next-hop IP address.

- Next-hop router is unreachable.

- Interface is down.

- Missing return route.

Step 5: Optimize the Routing Table

Ensure that:

- The next-hop router 10.0.0.3 is reachable.

- Interfaces are active.

- Return routes are correctly configured.

This improves packet forwarding efficiency.

Step 6: Route Used if No Matching Destination Exists

If no routing entry matches the destination, the router forwards the packet using the Default Route (10.0.0.1).