SIMATS ENGINEERING

ASSESSMENT TOOL 1

Experiments

COURSE NAME: Computer Networks COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP,

and DNS under varying conditions

Assessment Tool Weightage: 40%

SOFTWARE: CISCO PACKET TRACER, WIRESHARK

EXPERIMENTS

QUESTIONS:4 Total Marks:40

1. Design a simple LAN consisting of two PCs connected through a switch using

Cisco Packet Tracer. Configure appropriate IP addresses for both hosts and

verify connectivity using the ping command. Simultaneously capture the packet

exchange using Wireshark. Identify the Ethernet frame fields such as Source

MAC Address, Destination MAC Address, EtherType, and Frame Check

Sequence (FCS). Explain how the Data Link layer uses MAC addressing to

ensure successful frame delivery within the local network.

2. Create a network topology consisting of two LANs connected through a router

in Cisco Packet Tracer. Configure IPv4 addressing, subnet masks, and default

gateways for all devices, and verify end-to-end communication using the ping

command. Capture the ICMP packets using Wireshark and analyze the IPv4

header fields, including Source IP Address, Destination IP Address, Time-To-

Live (TTL), Protocol Number, and Header Length. Explain the role of the

Network layer in forwarding packets between different networks.

3. Construct a local area network with multiple hosts connected through a switch in

Cisco Packet Tracer. Clear the ARP cache on one of the hosts and initiate

communication with another host using the ping command. Capture the

communication in Wireshark and identify the ARP Request and ARP Reply

packets. Analyze the sender and target MAC/IP addresses and explain how the

Address Resolution Protocol resolves logical IP addresses into physical MAC

addresses before data transmission.

4. Develop a simple network consisting of two PCs connected through a router in

Cisco Packet Tracer. Configure the required IP addresses and test connectivity

using the ping command. Capture the ICMP traffic using Wireshark and identify

the Echo Request and Echo Reply messages. Examine important ICMP fields

such as Type, Code, Identifier, and Sequence Number, and calculate the Round

Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and

connectivity verification.

5. Create a client-server network in Cisco Packet Tracer by configuring a web

server and a client PC. Enable the HTTP service on the server and access the

hosted webpage from the client using a web browser. Capture the

communication in Wireshark and identify the TCP three-way handshake,

followed by the HTTP GET request and HTTP response messages. Analyze the

application-layer communication and explain how TCP provides reliable

delivery for HTTP traffic through connection establishment, acknowledgments,

and ordered data transfer.

RUBRICS FOR CALCULATION

QUESTION 1

Topology

IP Address

SIMULATION

2,SIMULATION: