SIMATS ENGINEERING

ASSESSMENT TOOL 2

NAME: SIVARANGJENII B

REG NO: 192521375

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network

infrastructure for its academic and administrative departments. The network

administrator has been allocated the IP address block 192.168.100.0/24. Based

on the expected number of devices, the following subnet masks have already

been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the

following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid host IP address range for each subnet.

3. Calculate the number of usable host addresses available in each subnet.

4. Determine the remaining unused IP address range within the given

network block.

2. A multinational software company is expanding its headquarters and has been

assigned the private IP block 10.10.0.0/16. Different departments require

different subnet sizes based on the number of employees. The network

administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid range of host IP addresses.

3. Calculate the number of usable hosts in each subnet.

4. Identify the unused IP address range remaining within the allocated IP

block.

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to

support thousands of connected medical devices and IoT sensors. The hospital

has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

1. Convert the given IPv6 address into its compressed notation.

2. Expand the compressed address back into its full hexadecimal

representation.

3. Identify the network prefix using the /64 prefix length.

4. Calculate the total number of host addresses supported within the

subnet.

5. the total number of possible host addresses available within this network.

4. An Internet Service Provider (ISP) connects multiple customer networks

through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

1. The destination subnet to which the packet belongs.

2. The matching routing table entry.

3. The next-hop network/interface used to forward the packet.

4. The default route that should be used if the destination does not match

any routing table entry.

5. A company establishes two office departments connected through a Layer-3

router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each LAN.

2. Identify the valid host IP address range.

3. Suggest suitable IP addresses for three computers in each LAN.

4. Identify the default gateway that hosts in each LAN should use for

communication.

Question 1

Given

IP Address Block = 192.168.20.0/24

Lab A = /26

Lab B = /27

Lab C = /28

Step 1 : Lab A (/26)

Subnet Mask = 255.255.255.192

Block Size

Formula:

Block Size = 256 − Last Octet of Subnet Mask = 256 – 192= 64

Subnet Increment = 0, 64, 128, 192

Network Address = 192.168.20.0

Broadcast Address = Next Network – 1 = 192.168.20.63

Valid Host IP Address Range

First Host = 192.168.20.1

Last Host = 192.168.20.62

Usable Host Addresses Host Bits = 32 – 26 = 6

Total Addresses = 2⁶ = 64

Usable Hosts = 64 – 2 = 62

Step 2 : Lab B (/27)

Next Available Network = 192.168.20.64

Subnet Mask = 255.255.255.224

Block Size

Formula:

Block Size = 256 − Last Octet of Subnet Mask = 256 – 224 = 32

Subnet Increment = 64, 96, 128, 160, 192, 224

Network Address = 192.168.20.64

Broadcast Address = Next Network – 1 = 192.168.20.96 – 1 = 192.168.20.95

Valid Host IP Address Range

First Host = 192.168.20.65

Last Host = 192.168.20.94

Usable Host Addresses

Host Bits = 32 – 27 = 5

Total Addresses = 2⁵ = 32

Usable Hosts = 32 – 2 = 30

Step 3 : Lab C (/28)

Next Available Network = 192.168.20.96

Subnet Mask = 255.255.255.240

Block Size

Formula:

Block Size = 256 − Last Octet of Subnet Mask

= 256 − 240

= 16

Subnet Increment

= 96, 112, 128, 144, 160, 176, 192, 208, 224, 240

Network Address

= 192.168.20.96

Broadcast Address

= Next Network − 1

= 192.168.20.112 − 1

= 192.168.20.111

Valid Host IP Address Range

First Host = 192.168.20.97

Last Host = 192.168.20.110

Usable Host Addresses

Host Bits = 32 − 28

= 4

Total Addresses = 2⁴

= 16

Usable Hosts = 16 − 2

= 14

Answer for Task 1 : Determine the Network Address and Broadcast Address

Lab A

Network Address = 192.168.20.0

Broadcast Address = 192.168.20.63

Lab B

Network Address = 192.168.20.64

Broadcast Address = 192.168.20.95

Lab C

Network Address = 192.168.20.96

Broadcast Address = 192.168.20.111

Answer for Task 2 : Identify the Valid Host IP Address Range

Lab A

First Host = 192.168.20.1

Last Host = 192.168.20.62

Lab B

First Host = 192.168.20.65

Last Host = 192.168.20.94

Lab C

First Host = 192.168.20.97

Last Host = 192.168.20.110

Answer for Task 3 : Calculate the Number of Usable Host Addresses

Lab A

Usable Hosts = 62

Lab B

Usable Hosts = 30

Lab C

Usable Hosts = 14

Question 2

Given

System 1 IP Address = 192.168.1.130

System 2 IP Address = 192.168.1.190

Subnet Mask = 255.255.255.192 (/26)

Step 1 : Find the Block Size

Subnet Mask = 255.255.255.192

Block Size

Formula:

Block Size = 256 − Last Octet of Subnet Mask

= 256 − 192

= 64

Subnet Increment

= 0, 64, 128, 192

The subnet ranges are:

- 192.168.1.0 – 192.168.1.63

- 192.168.1.64 – 192.168.1.127

- 192.168.1.128 – 192.168.1.191

- 192.168.1.192 – 192.168.1.255

Step 2 : Determine the Subnet for System 1

IP Address = 192.168.1.130

192.168.1.130 lies between 192.168.1.128 and 192.168.1.191

Network Address

= 192.168.1.128

Broadcast Address

= 192.168.1.191

Valid Host IP Address Range

First Host = 192.168.1.129

Last Host = 192.168.1.190

Check the Given IP Address

192.168.1.130 lies between

192.168.1.129 and 192.168.1.190

Therefore,

192.168.1.130 belongs to this subnet.

Step 3 : Determine the Subnet for System 2

IP Address = 192.168.1.190

192.168.1.190 lies between 192.168.1.128 and 192.168.1.191

Network Address

= 192.168.1.128

Broadcast Address

= 192.168.1.191

Valid Host IP Address Range

First Host = 192.168.1.129

Last Host = 192.168.1.190

Check the Given IP Address

192.168.1.190 lies between

192.168.1.129 and 192.168.1.190

Therefore,

192.168.1.190 belongs to this subnet.

Step 4 : Calculate the Number of Usable Host Addresses

Host Bits

= 32 − 26

= 6

Total Addresses

= 2⁶

= 64

Usable Hosts

= 64 − 2

= 62

Answer for Task 1 : Determine the Network Address and Broadcast Address

System 1

Network Address = 192.168.1.128

Broadcast Address = 192.168.1.191

System 2

Network Address = 192.168.1.128

Broadcast Address = 192.168.1.191

Answer for Task 2 : Identify the Valid Host IP Address Range

System 1

First Host = 192.168.1.129

Last Host = 192.168.1.190

System 2

First Host = 192.168.1.129

Last Host = 192.168.1.190

Answer for Task 3 : Verify Whether the IP Addresses Belong to the Subnet

System 1

192.168.1.130 belongs to the subnet 192.168.1.128/26.

System 2

192.168.1.190 belongs to the subnet 192.168.1.128/26.

Answer for Task 4 : Number of Usable Host Addresses

Usable Hosts = 62

Question 3

Given

IP Address Block = 172.16.0.0/16

HR Department = /25

Sales Department = /26

IT Department = /27

Admin Department = /28

Step 1 : HR Department (/25)

Subnet Mask = 255.255.255.128

Block Size

Formula:

Block Size = 256 − Last Octet of Subnet Mask

= 256 − 128

= 128

Network Address

= 172.16.0.0

Broadcast Address

= Next Network − 1

= 172.16.0.128 − 1

= 172.16.0.127

Valid Host IP Address Range

First Host = 172.16.0.1

Last Host = 172.16.0.126

Usable Host Addresses

Host Bits = 32 − 25

= 7

Total Addresses = 2⁷

= 128

Usable Hosts = 128 − 2

= 126

Step 2 : Sales Department (/26)

Next Available Network = 172.16.0.128

Subnet Mask = 255.255.255.192

Block Size

= 256 − 192

= 64

Network Address

= 172.16.0.128

Broadcast Address

= Next Network − 1

= 172.16.0.192 − 1

= 172.16.0.191

Valid Host IP Address Range

First Host = 172.16.0.129

Last Host = 172.16.0.190

Usable Host Addresses

Host Bits = 32 − 26

= 6

Total Addresses = 2⁶

= 64

Usable Hosts = 64 − 2

= 62

Step 3 : IT Department (/27)

Next Available Network = 172.16.0.192

Subnet Mask = 255.255.255.224

Block Size

= 256 − 224

= 32

Network Address

= 172.16.0.192

Broadcast Address

= Next Network − 1

= 172.16.0.224 − 1

= 172.16.0.223

Valid Host IP Address Range

First Host = 172.16.0.193

Last Host = 172.16.0.222

Usable Host Addresses

Host Bits = 32 − 27

= 5

Total Addresses = 2⁵

= 32

Usable Hosts = 32 − 2

= 30

Step 4 : Admin Department (/28)

Next Available Network = 172.16.0.224

Subnet Mask = 255.255.255.240

Block Size

= 256 − 240

= 16

Network Address

= 172.16.0.224

Broadcast Address

= Next Network − 1

= 172.16.0.240 − 1

= 172.16.0.239

Valid Host IP Address Range

First Host = 172.16.0.225

Last Host = 172.16.0.238

Usable Host Addresses

Host Bits = 32 − 28

= 4

Total Addresses = 2⁴

= 16

Usable Hosts = 16 − 2

= 14

Step 5 : Remaining Unused IP Address Range

HR Department

= 172.16.0.0 – 172.16.0.127

Sales Department

= 172.16.0.128 – 172.16.0.191

IT Department

= 172.16.0.192 – 172.16.0.223

Admin Department

= 172.16.0.224 – 172.16.0.239

Remaining Unused IP Address Range

= 172.16.0.240 – 172.16.255.255

Answer for Task 1 : Determine the Network Address and Broadcast Address

HR

Network Address = 172.16.0.0

Broadcast Address = 172.16.0.127

Sales

Network Address = 172.16.0.128

Broadcast Address = 172.16.0.191

IT

Network Address = 172.16.0.192

Broadcast Address = 172.16.0.223

Admin

Network Address = 172.16.0.224

Broadcast Address = 172.16.0.239

Answer for Task 2 : Identify the Valid Host IP Address Range

HR

First Host = 172.16.0.1

Last Host = 172.16.0.126

Sales

First Host = 172.16.0.129

Last Host = 172.16.0.190

IT

First Host = 172.16.0.193

Last Host = 172.16.0.222

Admin

First Host = 172.16.0.225

Last Host = 172.16.0.238

Answer for Task 3 : Calculate the Number of Usable Host Addresses

HR = 126

Sales = 62

IT = 30

Admin = 14

Answer for Task 4 : Remaining Unused IP Address Range

172.16.0.240 – 172.16.255.255

4 and 5

Question 4

Given

Network Address = 172.16.10.0/24

Subnet Mask = /27

IP Address 1 = 172.16.10.78

IP Address 2 = 172.16.10.95

Step 1 : Find the Subnet Mask

Subnet Mask

= 255.255.255.224

Step 2 : Calculate the Block Size

Formula:

Block Size = 256 − Last Octet of Subnet Mask

= 256 − 224

= 32

Subnet Increment

= 0, 32, 64, 96, 128, 160, 192, 224

Step 3 : Calculate the Total Number of Subnets

Original Prefix

= /24

New Prefix

= /27

Borrowed Bits

= 27 − 24

= 3

Formula:

Total Subnets = 2^(Borrowed Bits)

= 2³

= 8 Subnets

Step 4 : Calculate the Number of Usable Hosts

Host Bits

= 32 − 27

= 5

Total Addresses

= 2⁵

= 32

Usable Hosts

= 32 − 2

= 30 Hosts

Step 5 : Determine the Subnet for IP Address 172.16.10.78

The subnet ranges are:

172.16.10.0 – 172.16.10.31

172.16.10.32 – 172.16.10.63

172.16.10.64 – 172.16.10.95

172.16.10.96 – 172.16.10.127

Since 172.16.10.78 lies between 172.16.10.64 and 172.16.10.95,

Network Address

= 172.16.10.64

Broadcast Address

= 172.16.10.95

Valid Host IP Address Range

First Host = 172.16.10.65

Last Host = 172.16.10.94

Step 6 : Verify Whether 172.16.10.95 Belongs to the Same Subnet

Given IP Address

= 172.16.10.95

172.16.10.95 is equal to the Broadcast Address of the subnet.

Therefore,

172.16.10.95 belongs to the same subnet, but it cannot be assigned to any host because it is the Broadcast Address.

Answer for Task 1 : Total Number of Subnets

Total Subnets = 8

Answer for Task 2 : Number of Usable Hosts per Subnet

Usable Hosts = 30

Answer for Task 3 : Determine the Subnet for 172.16.10.78

Network Address = 172.16.10.64

Broadcast Address = 172.16.10.95

Valid Host Range = 172.16.10.65 – 172.16.10.94

Answer for Task 4 : Verify 172.16.10.95

172.16.10.95 belongs to the same subnet, but it is the Broadcast Address and cannot be assigned to a host.

Question 5

Given

IPv6 Address

= 2001:0db8:0000:0000:0000:ff00:0042:8329

Prefix Length

= /64

Step 1 : Compress the IPv6 Address

Rule 1

Remove the leading zeros from each block.

2001:0db8:0000:0000:0000:ff00:0042:8329

↓

2001:db8:0:0:0:ff00:42:8329

Rule 2

Replace consecutive blocks of 0 with :: (only once).

Compressed IPv6 Address

= 2001:db8::ff00:42:8329

Step 2 : Expand the Compressed IPv6 Address

Compressed Address

= 2001:db8::ff00:42:8329

Replace :: with three missing 0000 blocks.

Expanded IPv6 Address

= 2001:0db8:0000:0000:0000:ff00:0042:8329

Step 3 : Determine the Network Prefix

Prefix Length

= /64

The first 64 bits (first four hexadecimal blocks) represent the network.

Network Prefix

= 2001:0db8:0000:0000::/64

Compressed Form

= 2001:db8::/64

Step 4 : Calculate the Number of Host Bits

IPv6 Address Length

= 128 bits

Prefix Length

= 64 bits

Host Bits

= 128 − 64

= 64 bits

Step 5 : Calculate the Total Number of Host Addresses

Formula:

Total Host Addresses = 2^(Host Bits)

= 2⁶⁴

= 18,446,744,073,709,551,616

Approximately

= 18.4 Quintillion Host Addresses

Answer for Task 1 : Compressed IPv6 Address

2001:db8::ff00:42:8329

Answer for Task 2 : Expanded IPv6 Address

2001:0db8:0000:0000:0000:ff00:0042:8329

Answer for Task 3 : Network Prefix

2001:0db8:0000:0000::/64

(Compressed Form: 2001:db8::/64)

Answer for Task 4 : Total Number of Host Bits

64 Bits

Answer for Task 5 : Total Number of Possible Host Addresses

2⁶⁴ = 18,446,744,073,709,551,616 (Approximately 18.4 Quintillion Host Addresses)